Microprocessors, Microcontrollers and Assembly Language Lab

Lab Report - 4

Course Title	: Microprocessors, Microcontrollers and Assembly

Language Lab

Course Code	: CSE0611324

Submitted to:

| Teacher’s Name | Designation |
| --- | --- |
| Tahmina Yeasmin Lima | Lecturer |
| Abrar Mahbub Tanim | Lecturer |

Submitted by:

Student Name		: Gaus Saraf Murady

Student ID      		: 0432410005101088

Semester        		: Autumn 2026

Batch              		: 55

Department     		: CSE

Date of Submission: 06.09.26

Experiment No	: 4

Experiment Name: Conversion of lowercase letter to uppercase letter and vise versa

Introduction:

In this lab, we worked with Alphabets to convert Small Letter inputs to Capital Letter  counterparts. The core logic is the conversion of ASCII values sub al, 20h or sub al, 32 so the output comes out as the Capital Letter.

For our assignment, in addition to the core logic, we had to use the reverse logic (Capital→Small conversion) with JBE and CMP commands (If-Else equivalent of Programming Languages).

Code:

.model small .stack 100h .data msg1 db "Input: $" msg2 db "Output : $" char db ? .code main proc mov ax, @data mov ds,ax ; INPUT mov dx, offset msg1 mov ah, 09h int 21h mov ah, 01h int 21h ; sub al, 20h ; WILL BE ADD / SUM only after determining Capital / Small mov char, al ; ///////// mov ah, 02h mov dl,10 int 21h mov ah, 02h mov dl, 13 int 21h ; ///////// ; OUTPUT mov dx, offset msg2 mov ah, 09h int 21h ; IF BLOCK (if Capital, make Small) mov al, char ; storing `char` value to AL to compare with 61h cmp al, 60h ; comparing with the character before 'a' (" ' ") jbe to_SMALL ; ELSE BLOCK (if Small, make Capital) sub al, 20h jmp to_Output to_SMALL: add al, 20h to_Output: mov char, al mov ah, 02h mov dl, char int 21h exit: mov ah, 4ch int 21h main endp end main

Output:

Discussions:

The relevant notes have been documented at: Microprocessor_Assembly_Lab/doc/Assignments/Learning/A_4-theory.md at main · b1tranger/Microprocessor_Assembly_Lab

Also see: (From which I learned that we are working with flags instead of actual value changes (If-Else checks).

Conclusion:

I have successfully completed the assignment. While working on it, I had to study the Procedure Calling and Level Jumping to decide which one to use for this assignment.